DEPARTMENT OF  CHEMICAL ENGINEERING

B. I. T.  SINDRI, DHANBAD – 828 123

Modernization / replacement of laboratories equipments i.e. list of  equipments.

| Sl. No. | Items with descriptions – Hardware | Qty. |
| --- | --- | --- |
| 1 | Composite Wall Apparatus | Per unit |
| 2 | Wilfley Table | Per unit |
| 3 | Tray Dryer | Per unit |
| 4 | Air Classifier | Per unit |
| 5 | Dorr Thickener | Per unit |
| 6 | Heat Transfer in Natural Convection | Per unit |
| 7 | Penetrometer | Per unit |
| 8 | Smoke Point Apparatus | Per unit |
| 9 | Analytical Lab Balance | Per unit |
| 10 | Electronic Weighing Machine | Per unit |
| 11 | Thermocouple | Per unit |
| 12 | Potentiometric Displacement Sensor | Per unit |
| 13 | Dead-Weight Tester | Per unit |
| 14 | Water-Bath | Per unit |
| 15 | Digital Conductivity Meter | Per unit |
| 16 | Glass Rotameter for AIR | Per unit |
| 17 | Glass Rotameter for Water | Per unit |
| 18 | Glass Rotameter for Air | Per unit |
| 19 | U-Tube Manometer | Per unit |
| 20 | Digital Weighing Scale | Per unit |

Specifications

- Composite Wall Apparatus

Product description:

Technical Specifications:

- Slab assembly arranged symmetrically on both sides of heater.

- Slab Material : Slab Size

- Cast Iron : 250 mm dia. & 20mm thick.

- Bakelite : 250 mm dia. & 15 mm thick.

- Press Wood : 250 mm dia. & 12 mm thick.

- Heater : Nichrome Wire Heater.

- The slab assembly with front window of glass/acrylic.

- The whole set-up is ingeniously designed and schematically arranged on a powder-coated rigid structure.

Control Panel:

- Digital Voltmeter : 0-300 Volts.

- Digital Ammeter : 0-2 Amp.

- Variac : 0-230 V, 2Amp.

- Digital Temp. Indicator : 0-2000C, with multi-channel switch

- Temp Sensors : RTD PT-100 type.

- Standard make On/Off switch, Mains Indicator etc

- Wilfley Table

Product description.

UTILITIES REQUIRED :

Electricity Supply : Single Phase, 220 VAC Material to be separated.

TECHNICAL DETAILS: Wilfley Table Size : 800mm x 400mm Motor : ½ HP motor

Control Panel comprises of : Standard make on off switch, Mains Indicator etc.

- Tray Dryer

Product details: Capacity: 96 Trays Max Temperature : 100 Deg C Automatic Grade : Automatic Size : 80" W x 34" D x 77" H Operating Temperature From Ambient to 100 Deg C / 150 Deg C / 200 Deg C/ 250 Deg C /300 Deg C Heating Media : Electrical Heaters Trolley : 2 Nos. of trolleys having capacity of holding 48 Tray Size : 16" x 32" x 1 1/4 "

- Air Classifier

Specification

- Weight : 20-100t

- Length : 5-12m

- Liner : High Chrome steel

5. Dorr Thickener

Specification

Water Supply and Drain.

Electric Supply : Single phase, 220 VAC, 1.5 kW.

Floor Area 1m×1m.

Slurry (Paper pulp or CaCo3)

Technical Detail

Thickner  Material Stainless Steel Dia 400mm, Height 150mm

Thickner Agitator : SS impeller with SS Shaft coupled to FHP

motor with reduction gearbox.

Slurry Tank : Material Stainless Steel, Capacity 100 Ltrs.

Slurry Tank Agitator : Stainless Steel Impeller with SS Shaft coupled to

FHP Motor and Reduction Gear Box.

Slurry Feed Pump : Gear Pump with FHP motor.

Pining system : GI and PVC.

Control panel Comprises of : Standard make On/Off switch, Mains Indicator etc.

6. Heat Transfer in natural convection

Specifications

- Pipe – Brass pipe, 38mm. dia. (OD), 500mm. long, fitted with cartridge heater inside.

- Thermocouples are fitted along the length of pipe for temperature measurement – 7 nos.

- Enclosure 200mm. × 200mm. × 800mm. Size, with one side of Perspex sheet.

- Measurements & Controls.

- A dimmer stat for heater input control.

- Voltmeter and Ammeter for heater input measurement.

- Multi channel digital temperature indicator.

A technical manual accompanies the unit.

7. Penetrometer

DESCRIPTION

This apparatus is used for penetration test on a wide variety of materials such as greases, petroleum, bitumen, tax, wax, polish, rubber asphalts and pharmaceutical creams. A given force is applied over a given period of times and the depression is made in the sample which is measured in tenths of a millimeter and expressed as penetration. It consists of a head support mounted on a vertical rod assembly provides fine adjustment of needle tip to sample and in corporate a slipping clutch mechanism which make reading of penetration and subsequent resetting a sample and accurate operation. The dial graduate from 0-400 in one tenth millimeter sub division.

Penetrometer Apparatus with containers and weights having

- Penetration cone hollow type

- Wax worker hand operated

- Wax Cutter

- Transfer dish

Specifications

1. With containers & weights having penetration needle

2. With containers & weights having penetration cone, hollow type

8. Smoke Point Apparatus

DESCRIPTION

An instrument for determining the smoke point of kerosine and aviation turbine fuel by measuring the maximum flame height prior to smoke formation and comparing the results with those of known standards. The smoke point gives an indication of aromatic content and radiant heat transfer characteristics and has good correlation with ASTM D1740 (Luminometer number).

Comprises a tripod stand, lamp body, and chimney. The measuring scale is viewed through a curved glass window on the front of the instrument. A mirror (10450-0) can be attached to the chimney to aid smoke detection.

SPECIFICATIONS

Size (H x W x D) :47 x 21 x 19cm

Weight: 2.6 kg

- ANALYTICAL LAB BALANCE

| Capacity*Readability | 120 g x 0.1 mg |
| --- | --- |
| Repeatability (std deviation) | ±0.1 mg |
| Linearity | ± 0.2 mg |
| Response Time (average) | 2.5 seconds |
| Tare Range | FULL to Capacity |
| Selectable Application Programs | Toggle between 5 weight units, weigh average, calculation, dynamic animal weighing/averaging, net-total formulation, percent weighing, totalizing, density determination and counting |
| Display Update | 0.1 - 0.4 seconds |
| Display | LCD with backlit |
| Operating Temperature Range | 50° F to 86° F (+10° C to +30° C) |
| Allowable Ambient Operating Temperature | 41° F to 104° F (+5° C to +40° C) |
| Sensitivity Drift | ±2 ppm / °C |
| Motorized Internal Calibration | YES |
| External Calibration | 100 g Optional |
| Built-in RS-232 Interface | YES |
| Weighing Below | A port for a hanger for weighing below the balance is available (requires purchase of optional accessory - 3 mm weigh below hook). You can attach a sample using a suspension wire, for example. Common applications for weighing below the balance includes density determination. |
| Standard Equipment | AC Adapter, Sliding-door draft shield, Shield ring, Shield plate, Motorized internal calibration, Dust Cover |

- ELECTRONIC WEIGHING MACHINE

Capacity: 0.1-10 kg

Accuracy: 0.01-0.1 kg

- THERMOCOUPLE

Sheath OD: 1/16 inch or1/8 inch

Measuring Junction Grounded

Response time: 0.5

| Calibration Type | Conductors |  | Temperature Range | Limits of Error | Extention wire jacket colour | Colour coding |
| --- | --- | --- | --- | --- | --- | --- |
|  | Positive | Negative |  |  |  |  |
| T | Copper | Constantan | -200 °C to 350 °C | ±1 °C | Blue | Blue+ Red - |
| J | Iron | Constantan | 0 °C -750 °C | ±1 °C | Black | White+ Red- |
| E | Chromel | Constantan | -200 °C to 900 °C | ±1 °C | Yellow | Yellow + Red- |

- POTENTIOMETRIC DISPLACEMENT SENSOR

## Product Description

The measurement of position and displacement of physical objects is essential for many applications such as process feedback control, performance evaluation, transportation traffic control, robotics, and security systems just to name a few. By position, we mean the determination of the objects coordinates (linear or angular) with respect to a selected reference.

Displacement means moving from one position to another for a specific distance or angle. In other words, a displacement is measured when an object is referenced to its own prior position rather than to another reference. Potentiometric Displacement Sensor, is designed to learn details of Displacement Sensor.

The simplest type of Displacement Sensor involves the action of displacement in moving the wiper of a potentiometer. This device then converts linear or angular motion into a changing resistance that may be converted directly to voltage and/ or current signals.

## Product Features

- Self-contained and easy to operate.

- Sensitive, Linear, Stable & Accurate.

- Built in DC Power Supplies.

- Micrometer for displacement measurement.

- Calibrated dials for output position.

- Servo Potentiometers with full 360

- Online Product Tutorials

## Scope of Learning

### Study and measurement of linear displacement.

### Study and measurement of angular displacement

- DEAD-WEIGHT TESTER

| Range | 1 to 400 kg/cm² The range can be extended to lower or higher pressure ranges with the procurement of a Piston Cylinder Unit and weight set only. The same base can be used for fitment of all the DH-Budenberg's hydraulic Piston Cylinder units. Pressures up to 1200 bar can be achieved using the same base. |
| --- | --- |
| Accuracy | Range 1 to 6 kg/cm² : 0.025% Range 6 to 70 kg/cm² : 0.02% Range 70 to 400 kg/cm² : 0.015% |
| Repeatability | < 10 PPM |
| Sensitivity | < 10 PPM |
| Temperature Coefficient of PCU | 16.5 x 10-6 / degree centigrade |
| Software | Software package supplied to provide corrections for local gravity, true piston cylinder diameter and temperature. |
| Calibration Certification | Calibration certificate Traceable |
| Weights | The weights are a set of 8 manufactured from Austenitic non-magnetic 300 series Stainless Steel. The weights are all top loaded to reduce the requirement for an overhung weight carrier assembly. Weights are fully interchangeable with other sets of the same type, no serial number matching is required to the piston cylinder unit. Weights comprising : Piston 1/10 1 weight 9/90 2 weights 10/100 1 weight 5/50 2 weights 2/20 1 weight 1/10 1 weight 0.5/5 Weight sets can also be purchased in bar, Kpa or psi if required. |
| Piston Cylinder Unit | A dual area piston / cylinder in one compact housing with 1/8in2 and 1/80in2 areas. Changing between ranges is fully automated meaning that users have an extremely flexible instrument covering a very wide pressure range with one piston / cylinder unit and one weight set. Changeover from low to high range is achieved without the need to change weights or interchange piston cylinder units. The low pressure piston and high pressure cylinder is tungsten carbide. Both measuring pistons are located in a very robust housing with an auxiliary piston which prevents damage due to shock loadings during operation and during transportation. |
| Base | The base is universal and all DH-Budenberg hydraulic Piston Cylinder units can be interchanged in less than two minutes. Pressure control is provided by a dual area screwpress which allows fast priming at low pressures and easy generation of pressures up to 1200 bar with precision pressure control. The oil reservoir has a capacity up to 170 cc to allow a large volume system to be easily pressurised. The base is aluminium with an ABS cover having four levelling screws, four location mounts are also provided for bench mounting. The base can also be used as a comparator with the supply of a suitable adapter and test gauge. |
| Tool set and spare seals | Tool roll containing: Gauge stand, Stainless Steel connections for 1/4, 3/8 & 1/2 inch BSP, spirit level, 4 mounting feet, ½ litre container of hydraulic oil to ISOVG22, set of spares seals, hexagon wrench key, two spanners, gauge pointer remover and a gauge pointer punch. |
| Manual | Operating and maintenance manual |

- WATER-BATH

- Tank :High grade SS tank

- Capacity : 5 to 8 litres

- Temperature range : Ambient to 99° C

- Temperature accuracy: ± 0.2° C

- Temperature control : Digital PID control

- Timer : 0.1 Hour to 99.9 Hours & continuous

- Safety protection : User settable and fixed thermal cut-out

- Display : Digital display of temperature

- Alarm : Low level water alarm, safety shut down audible/visual alarm

- Operating voltage : 230 ±10VAC, 50 Hz.

- Tank design for easy cleaning and draining

- Steel holder for tubes (1.5/2 ml, 5ml, 15 ml) and Flasks (50, 100 & 250 ml)

The following items are required for the Fluidization Laboratory.

| Sl. No. | Item Name with specification | No. of Units | Unit price | Total price |
| --- | --- | --- | --- | --- |
| 15 | Digital Conductivity Meter (Range 0-20µs to 0- 200 mS, Solid and Liquid state, LSI Technology) |  |  |  |
| 16 | Glass Rotameter for AIR (Range :- 0- 100 lph, Sensitivity: +/-2% of F.S, Brass Insert, SS.304 Wetted Parts, BSP 3 bar Pressure) |  |  |  |
| 17 | Glass Rotameter for Water (Range :- 0- 100 lph, Sensitivity: +/-2% of F.S, Steel Float, SS.304 Wetted Parts, BSP 3 bar Pressure) |  |  |  |
| 18 | Glass Rotameter for AIR (Range :- 0- 500 lph, Sensitivity: +/-2% of F.S, Steel Float, SS.304 Wetted Parts, BSP 3 bar Pressure) |  |  |  |
| 19 | U Tube Manometer (Fluid: Water, Casing: M.S./Aluminum Casing from three sides and aluminum cover with the transparent acrylic front. Packing : Teflon/Neoprene For Glass Tube : 250 – 0- 250mm, 750-0-750mm) |  |  |  |
| 20 | Digital Weighing Scale (Pan Size : >80mm, Power Source: AC120/230V/DC9V, Readability : 0.0001g., Weighing Capacity:>2000g, Non Linearity : +/-0.0003g) |  |  |  |